Lockout Tagout Quiz Key

### Name:	 ___________________________ 	Date: ________

- The purpose of LOTO is to protect employees from: hazardous energy.

- Lockout Tagout is required for technician controlled cord powered devices.

True or False

- Keys for lockout devices can be passed from one employee to another.

True or False

- “Affected Employees” are employees that operate equipment or are in the vicinity of locked out equipment.

- Authorized Employees can isolate work areas from hazardous energy, Affected Employees cannot:               True or False

- It is the responsibility of authorized employees to inform affected employees prior to performing a lockout tagout operation.

8.	The six basic steps of a lockout tagout procedure are:

1.  Notify Employees			2.  Shutdown Equipment

3.  Isolate Energy				4.  Attach lockout device

5.  Release Energy				6. Verify Lockout

9.	Tags can be used as a substitution for locks:   True or False

- For maintenance work with more than one employee, each employee must have their own lock hanging on the isolation device:		True or False.