HAZWOPER: Awareness Level

Safety Training Handout

- As First Responders at the Awareness Level, you have a responsibility to

- Recognize hazardous substances

- Evacuate yourself and others

- Notify the proper authorities

- Secure the area

- Help determine the identity of the hazardous substance

- Hazardous chemicals that are spilled or released can

- Injure you or your co-workers

- Contaminate drinking water

- Cause a massive evacuation

- Kill innocent bystanders

- Signs of a chemical release or spill

- Liquid on the floor or tables

- Liquid dripping from container with hazardous materials

- Any odor that is unfamiliar to you

- Hissing or dripping sound

- Equipment that has loss of pressure or flow

- After you recognize a spill

- Warn others by yelling or setting off an alarm

- Turn off machines and evacuate

- Barricade the area to prevent unauthorized personnel from entering

- Stay upwind of the spill

- Don’t attempt to rescue anyone unless trained to do so

- Gather information about the spill for emergency response personnel

- Try to identify the hazardous chemical

- If exposure occurs

- Inform your supervisor and describe the chemical

- Flush eyes or skin for at least 15 minutes

- Seek immediate medical attention if you swallow chemicals