##### Hazard Communication

###### Safety Training Handout

# What is my “right to know?”

# You have the right and need to know about what chemicals you work with, what hazards they pose how to protect yourself from them.

# Why should I know the hazards of the chemical I work with?

# Chemicals present both physical hazards and health hazards.

# Physical hazards include dangers presented by flammable liquids, combustible liquids, compressed gases, explosives and water reactive materials.

# Hazardous chemicals can enter your body through inhalation, swallowing, contact with skin, or through the eyes, causing both short-term and long-term health hazards.

# In the case of acute/short-term health hazards, symptoms appear shortly after exposure to a high concentration. Illnesses include rashes, burns, dizziness and nausea.

# Chronic/long-term hazards are caused by repeat exposure to low concentrations of chemicals. Symptoms may not appear for years. Illnesses include sterility, cancer and damage to the central nervous system, lungs and kidneys.

# How can I learn about hazards and protect myself?

# Labels: Check labels prior to use for chemical name, information on appropriate PPE, safe handling practices and proper emergency response.  All containers must be labeled. Do not use materials from unlabeled containers.

# Material Safety Data Sheets (MSDS): Know where MSDS’s are stored for the chemicals you work with and check the MSDS prior to working with any unfamiliar chemicals. MSDS’s provide chemical names, company information, hazardous ingredients, physical characteristics, fire and explosion data, health hazard data, reactivity data, special precaution, PPE, safe handling and emergency response.

# Safe use practices: Use chemicals only as directed. Always use appropriate PPE and follow any special safe handling instructions. Never eat, drink, smoke or apply of cosmetics in area where chemicals are present.

# What should I do in the event of an emergency?

# Report leaks, spills, suspicious odors to supervisor immediately.

# Remove potential sources of ignition.

# Evacuate area.

# Stay away until given the “all clear.”

# Refer to MSDS for specific spill and leak information.

# Know what chemical or chemicals you were working with.

# What if I come into contact with a hazardous chemical?

# If chemicals get into your eyes, flush with running water for 15 minutes.

# If chemicals are swallowed, seek immediate medical attention.

# If chemicals contact your skin, wash thoroughly with soap and water.

# If hazardous vapors or fumes are inhaled, move immediately to fresh air.

# Report incidents to supervisor.