Ammonia Refrigeration Safety

# Quiz

- Name two areas where ammonia leaks can occur.

____________________________        _______________________________

- True or false?

Ammonia gas can kill you, but liquid (anhydrous) ammonia can’t.

- You must leave the work area if the atmosphere’s ammonia concentration is:

  - 5 parts per million (ppm)

  - 25 ppm

  - 50 ppm

- True or false?

Lockout/tagout rules apply to maintenance and repairs on refrigeration systems.

- True or false?

The Clean Water Act mandates ammonia leaks be repaired promptly.

- Which of the following is a true statement?

  - Static leaks are uncommon.

  - Pressure leaks don’t show up until pressure builds up in the system.

  - Vibration leaks are evident when the system’s not running.

- True or False?
Valves and stems shrink naturally over time and don’t usually cause problems.

- PPE, such as respirators, must be: _________________________________________

- Ammonia refrigeration compliance is handled by:

- OSHA

- OSHA and the EPA

- OSHA, the EPA and U.S. Health Department

10)	True or False?

Fluorescent leak detection is the easiest and most effective leak-detection method.

Name: __________________________________

Date: _______________________

Ammonia Refrigeration Safety

# Answers to the Quiz

1. Valves, lines, stems, tanks.

2. False. Overexposure to both forms of ammonia can kill you

3. B

4. True

5. False. The Clean Air Act mandates this

6. B

7. False. Deteriorated valves and stems lead to leaks

8. Easily accessible, in good condition, fit well

9. B

10. True