Forklift Safety

## Quiz

- True or False?
There are over 36,000 serious injuries and 100 deaths that result from forklift accidents 
per year.

- One in four forklift accidents is caused by:

- Faulty forklifts

- Inadequate training

- Tire blowouts

- None of the above

- While Operating a forklift, it is very important to stay aware of your:

_________________________________________________________________________

- Which of the following statements comparing forklifts and cars is false:

  - Forklifts are usually heavier than cars

  - Forklifts are steered by the back wheels

  - Forklifts are more stable than cars

  - None of the above

- When traveling with a load on inclines, it is best to:

  - Drive up and reverse down inclines

  - Reverse up and drive down inclines

  - Avoid inclines at all costs

- True or false?

Always carry loads tilted backward toward the forklift.

- True or false?

It’s best to carry loads 24 inches off the ground.

- True or false?

Even if a load is blocking your front view, never travel in reverse

- If your forklift is about to tip over, you should never:

  - Stay inside the cabin

  - Jump clear of the forklift

  - Keep your hands firmly on the steering wheel

  - Lean forward and away from the direction of the tip over

- True or false?
You should always drive at least three forklift lengths behind other vehicles

- Which of the following is not one of the three types of stability?

  - Longitudinal

  - Dynamic

  - Lateral

  - Longevity

- True or false?
The central point of balance, or the fulcrum, on a forklift is located under the front axle.

- When lifting a coworker on the forks of your truck, what should you never do?

  - Use a firmly secured safety platform with guardrails

  - Have an operator inside the forklift

  - Give lifted worker ability to shut off power to the forklift

  - Tilt forks backward so worker won’t fall

- List at least three things you should check prior to operating your forklift:

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

Name: _____________________________

Date: ______________________________

Forklift Safety

## Answers to Quiz

- True

- b

- Surroundings

- c

- a

- True

- False. It’s recommended to carry loads 6 inches off the ground.

- False. If a load is blocking your forward view, you should travel in reverse.

- b

- True

- d

- True

- d

- Types of answers to collect are cage, overhead guard, forks, tires, leaks, battery, horns, warning lights, brakes, seat brake, clutch, gauges, forward and reverse steering and hydraulic levers.